GO0226 ***********************************************************************

GO0226  DESIGNATION -  N 431

GO0226  PID         -  GO0226

GO0226  STATE/COUNTY-  NM/SAN JUAN

GO0226  COUNTRY     -  US

GO0226  USGS QUAD   -  FARMINGTON NORTH (1979)

GO0226

GO0226                         *CURRENT SURVEY CONTROL

GO0226  ______________________________________________________________________

GO0226* NAD 83(2011) POSITION- 36 45 30.53666(N) 108 09 27.87687(W)   ADJUSTED

GO0226* NAD 83(2011) ELLIP HT-  1644.195 (meters)        (06/27/12)   ADJUSTED

GO0226* NAD 83(2011) EPOCH   -  2010.00

GO0226* NAVD 88 ORTHO HEIGHT -  1665.329 (meters)     5463.67  (feet) ADJUSTED

GO0226  ______________________________________________________________________

GO0226  NAD 83(2011) X  - -1,594,755.498 (meters)                     COMP

GO0226  NAD 83(2011) Y  - -4,862,560.062 (meters)                     COMP

GO0226  NAD 83(2011) Z  -  3,796,937.751 (meters)                     COMP

GO0226  LAPLACE CORR    -          0.70  (seconds)                    DEFLEC12B

GO0226  GEOID HEIGHT    -        -21.134 (meters)                     GEOID12B

GO0226  DYNAMIC HEIGHT  -       1663.256 (meters)     5456.87  (feet) COMP

GO0226  MODELED GRAVITY -    979,329.0   (mgal)                       NAVD 88

GO0226

GO0226  VERT ORDER      -  FIRST     CLASS II

GO0226

GO0226  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

GO0226  Standards:

GO0226         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

GO0226            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

GO0226  -------------------------------------------------------------------

GO0226  NETWORK    0.47   1.08           0.19   0.19   0.55     -0.03728392

GO0226  -------------------------------------------------------------------

GO0226  Click here for local accuracies and other accuracy information.

GO0226

GO0226

GO0226.The horizontal coordinates were established by GPS observations

GO0226.and adjusted by the National Geodetic Survey in June 2012.

GO0226

GO0226.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

GO0226.been affixed to the stable North American tectonic plate. See

GO0226.NA2011 for more information.

GO0226

GO0226.The horizontal coordinates are valid at the epoch date displayed above

GO0226.which is a decimal equivalence of Year/Month/Day.

GO0226

GO0226.The orthometric height was determined by differential leveling and

GO0226.adjusted by the NATIONAL GEODETIC SURVEY

GO0226.in June 1991.

GO0226

GO0226.Significant digits in the geoid height do not necessarily reflect accuracy.

GO0226.GEOID12B height accuracy estimate available here.

GO0226

GO0226.The X, Y, and Z were computed from the position and the ellipsoidal ht.

GO0226

GO0226.The Laplace correction was computed from DEFLEC12B derived deflections.

GO0226

GO0226.The ellipsoidal height was determined by GPS observations

GO0226.and is referenced to NAD 83.

GO0226

GO0226.The dynamic height is computed by dividing the NAVD 88

GO0226.geopotential number by the normal gravity value computed on the

GO0226.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

GO0226.degrees latitude (g = 980.6199 gals.).

GO0226

GO0226.The modeled gravity was interpolated from observed gravity values.

GO0226

GO0226. The following values were computed from the NAD 83(2011) position.

GO0226

GO0226;                    North         East     Units Scale Factor Converg.

GO0226;SPC NM W     -   638,729.989   801,034.998   MT  0.99992700   -0 11 38.9

GO0226;SPC NM W     - 2,095,566.64  2,628,062.32   sFT  0.99992700   -0 11 38.9

GO0226;UTM  12      - 4,071,848.436   753,720.188   MT  1.00039315   +1 42 06.6

GO0226

GO0226!             -  Elev Factor  x  Scale Factor =   Combined Factor

GO0226!SPC NM W     -   0.99974203  x   0.99992700  =   0.99966905

GO0226!UTM  12      -   0.99974203  x   1.00039315  =   1.00013508

GO0226

GO0226                          SUPERSEDED SURVEY CONTROL

GO0226

GO0226  NAD 83(2007)-  36 45 30.53621(N)    108 09 27.87744(W) AD(2002.00) 0

GO0226  ELLIP H (02/10/07) 1644.213  (m)                       GP(2002.00)

GO0226  ELLIP H (08/10/01) 1644.234  (m)                       GP(       ) 3 2

GO0226  NAD 83(1992)-  36 45 30.53580(N)    108 09 27.87706(W) AD(       ) 1

GO0226  ELLIP H (03/20/97) 1644.228  (m)                       GP(       ) 3 1

GO0226  NAVD 88 (03/20/97) 1665.33   (m)         5463.7    (f) LEVELING    3

GO0226

GO0226.Superseded values are not recommended for survey control.

GO0226

GO0226.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

GO0226.See file dsdata.txt to determine how the superseded data were derived.

GO0226

GO0226_U.S. NATIONAL GRID SPATIAL ADDRESS: 12SYF5372071848(NAD 83)

GO0226

GO0226_MARKER: DB = BENCH MARK DISK

GO0226_SETTING: 66 = SET IN ROCK OUTCROP

GO0226_STAMPING: N 431 1983

GO0226_MARK LOGO: NGS

GO0226_MAGNETIC: N = NO MAGNETIC MATERIAL

GO0226_STABILITY: A = MOST RELIABLE AND EXPECTED TO HOLD

GO0226+STABILITY: POSITION/ELEVATION WELL

GO0226_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

GO0226+SATELLITE: SATELLITE OBSERVATIONS - July 31, 2001

GO0226

GO0226  HISTORY     - Date     Condition        Report By

GO0226  HISTORY     - 1983     MONUMENTED       NGS

GO0226  HISTORY     - 19960517 GOOD             CHANCE

GO0226  HISTORY     - 20010731 GOOD             COMPA

GO0226

GO0226                          STATION DESCRIPTION

GO0226

GO0226'DESCRIBED BY NATIONAL GEODETIC SURVEY 1983

GO0226'IN FARMINGTON.

GO0226'IN FARMINGTON, AT THE JUNCTION OF EAST MAIN STREET (U.S. HIGHWAY 550)

GO0226'AND 30TH STREET, IN THE CENTER OF A 10- BY 65-FOOT EXPOSED AREA OF

GO0226'OUTCROPPING BEDROCK, 147.5 METERS (483.9 FT) WEST OF THE CENTERLINE OF

GO0226'THE SOUTH BOUND LANES OF THE STREET, 35.7 METERS (117.1 FT) SOUTH OF

GO0226'THE CENTERLINE OF THE EAST BOUND LANES OF THE STREET, 35.4 METERS

GO0226'(116.1 FT) WEST OF A UTILITY POLE WITH TWO GUY WIRES AT A UTILITY LINE

GO0226'CROSSING THE STREET, 30.9 METERS (101.1 FT) SOUTH OF THE SOUTHWEST

GO0226'CORNER OF A MULTI-BOX CULVERT, 12.1 METERS (39.7 FT) SOUTHEAST OF THE

GO0226'NORTHWEST EDGE OF THE OUTCROP, 8.1 METERS (26.6 FT) NORTHWEST OF THE

GO0226'SOUTHEAST EDGE OF THE OUTCROP, AND 0.7 METER (2.3 FT) NORTHEAST OF THE

GO0226'SOUTHEAST EDGE OF THE OUTCROP.

GO0226'THE MARK IS 0.3 METERS  NW FROM A WITNESS POST.

GO0226'THE MARK IS 2.2 M ABOVE THE STREET.

GO0226

GO0226                          STATION RECOVERY (1996)

GO0226

GO0226'RECOVERY NOTE BY JE CHANCE AND ASSOCIATES 1996 (MFY)

GO0226'A NEW AND COMPLETE DESCRIPTION FOLLOWS THE STATION IS LOCATED IN

GO0226'FARMINGTON ALONG MAIN STREET (STATE HIGHWAY 550) .  OWNERSHIP --

GO0226'UNKNOWN TO REACH THE STATION FROM THE JUNCTION OF MAIN STREET (STATE

GO0226'HIGHWAY 550) AND 30TH STREET IN FARMINGTON, GO EAST ON 30TH STREET FOR

GO0226'147 M (482.3 FT) TO THE STATION ON THE LEFT THE STATION IS A BENCH

GO0226'MARK DISK SET INTO THE CENTER OF A 10- BY 65- FOOT EXPOSED AREA OF A

GO0226'ROCK OUTCROP.  THE STATION IS LOCATED 35.7 M (117.1 FT) SOUTH OF THE

GO0226'CENTERLINE OF THE EAST BOUND LANES OF 30TH STREET, 30.9 M (101.4 FT)

GO0226'SOUTH OF THE SOUTHWEST CORNER OF A MUTLI-BOX CULVERT, 12.1 M (39.7 FT)

GO0226'SOUTHEAST OF THE NORTHWEST EDGE OF THE OUTCROP, 8.1 M (26.6 FT)

GO0226'NORTHWEST OF THE SOUTHEAST EDGE OF THE OUTCROP, 0.7 M (2.3 FT)

GO0226'NORTHEAST OF THE SOUTHEAST EDGE OF THE OUTCROP, 2.2 M (7.2 FT) ABOVE

GO0226'THE 30TH STREET, AND 0.3 M (1.0 FT) NORTHWEST FROM A WITNESS POST

GO0226

GO0226                          STATION RECOVERY (2001)

GO0226

GO0226'RECOVERY NOTE BY COMPASSCOM INC 2001 (RL)

GO0226'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = AUGUST 29, 2016

GO0210 ***********************************************************************

GO0210  DESIGNATION -  E 432

GO0210  PID         -  GO0210

GO0210  STATE/COUNTY-  NM/SAN JUAN

GO0210  COUNTRY     -  US

GO0210  USGS QUAD   -  KIRTLAND (1979)

GO0210

GO0210                         *CURRENT SURVEY CONTROL

GO0210  ______________________________________________________________________

GO0210* NAD 83(2011) POSITION- 36 44 45.43458(N) 108 22 07.08425(W)   ADJUSTED

GO0210* NAD 83(2011) ELLIP HT-  1575.221 (meters)        (06/27/12)   ADJUSTED

GO0210* NAD 83(2011) EPOCH   -  2010.00

GO0210* NAVD 88 ORTHO HEIGHT -  1596.394 (meters)     5237.50  (feet) ADJUSTED

GO0210  ______________________________________________________________________

GO0210  NAD 83(2011) X  - -1,612,887.287 (meters)                     COMP

GO0210  NAD 83(2011) Y  - -4,857,394.519 (meters)                     COMP

GO0210  NAD 83(2011) Z  -  3,795,782.239 (meters)                     COMP

GO0210  LAPLACE CORR    -          0.28  (seconds)                    DEFLEC12B

GO0210  GEOID HEIGHT    -        -21.198 (meters)                     GEOID12B

GO0210  DYNAMIC HEIGHT  -       1594.447 (meters)     5231.11  (feet) COMP

GO0210  MODELED GRAVITY -    979,356.3   (mgal)                       NAVD 88

GO0210

GO0210  VERT ORDER      -  FIRST     CLASS II

GO0210

GO0210  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

GO0210  Standards:

GO0210         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

GO0210            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

GO0210  -------------------------------------------------------------------

GO0210  NETWORK    0.47   1.08           0.19   0.19   0.55     -0.03755307

GO0210  -------------------------------------------------------------------

GO0210  Click here for local accuracies and other accuracy information.

GO0210

GO0210

GO0210.The horizontal coordinates were established by GPS observations

GO0210.and adjusted by the National Geodetic Survey in June 2012.

GO0210

GO0210.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

GO0210.been affixed to the stable North American tectonic plate. See

GO0210.NA2011 for more information.

GO0210

GO0210.The horizontal coordinates are valid at the epoch date displayed above

GO0210.which is a decimal equivalence of Year/Month/Day.

GO0210

GO0210.The orthometric height was determined by differential leveling and

GO0210.adjusted by the NATIONAL GEODETIC SURVEY

GO0210.in June 1991.

GO0210

GO0210.Significant digits in the geoid height do not necessarily reflect accuracy.

GO0210.GEOID12B height accuracy estimate available here.

GO0210

GO0210.The X, Y, and Z were computed from the position and the ellipsoidal ht.

GO0210

GO0210.The Laplace correction was computed from DEFLEC12B derived deflections.

GO0210

GO0210.The ellipsoidal height was determined by GPS observations

GO0210.and is referenced to NAD 83.

GO0210

GO0210.The dynamic height is computed by dividing the NAVD 88

GO0210.geopotential number by the normal gravity value computed on the

GO0210.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

GO0210.degrees latitude (g = 980.6199 gals.).

GO0210

GO0210.The modeled gravity was interpolated from observed gravity values.

GO0210

GO0210. The following values were computed from the NAD 83(2011) position.

GO0210

GO0210;                    North         East     Units Scale Factor Converg.

GO0210;SPC NM W     -   637,424.320   782,197.681   MT  0.99994481   -0 19 12.9

GO0210;SPC NM W     - 2,091,282.96  2,566,260.23   sFT  0.99994481   -0 19 12.9

GO0210;UTM  12      - 4,069,919.597   734,928.843   MT  1.00028000   +1 34 29.9

GO0210

GO0210!             -  Elev Factor  x  Scale Factor =   Combined Factor

GO0210!SPC NM W     -   0.99975285  x   0.99994481  =   0.99969768

GO0210!UTM  12      -   0.99975285  x   1.00028000  =   1.00003278

GO0210

GO0210                          SUPERSEDED SURVEY CONTROL

GO0210

GO0210  NAD 83(2007)-  36 44 45.43413(N)    108 22 07.08482(W) AD(2002.00) 0

GO0210  ELLIP H (02/10/07) 1575.238  (m)                       GP(2002.00)

GO0210  ELLIP H (08/10/01) 1575.259  (m)                       GP(       ) 3 2

GO0210  NAD 83(1992)-  36 44 45.43369(N)    108 22 07.08437(W) AD(       ) 1

GO0210  ELLIP H (03/20/97) 1575.253  (m)                       GP(       ) 3 1

GO0210  NAVD 88 (03/20/97) 1596.39   (m)         5237.5    (f) LEVELING    3

GO0210

GO0210.Superseded values are not recommended for survey control.

GO0210

GO0210.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

GO0210.See file dsdata.txt to determine how the superseded data were derived.

GO0210

GO0210_U.S. NATIONAL GRID SPATIAL ADDRESS: 12SYF3492869919(NAD 83)

GO0210

GO0210_MARKER: I = METAL ROD

GO0210_SETTING: 49 = STAINLESS STEEL ROD W/O SLEEVE (10 FT.+)

GO0210_STAMPING: E 432 1983

GO0210_MARK LOGO: NGS

GO0210_PROJECTION: FLUSH

GO0210_MAGNETIC: N = NO MAGNETIC MATERIAL

GO0210_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

GO0210_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

GO0210+SATELLITE: SATELLITE OBSERVATIONS - March 20, 2006

GO0210_ROD/PIPE-DEPTH: 7.3  meters

GO0210

GO0210  HISTORY     - Date     Condition        Report By

GO0210  HISTORY     - 1983     MONUMENTED       NGS

GO0210  HISTORY     - 19960106 GOOD             CHANCE

GO0210  HISTORY     - 20060320 GOOD             BOR

GO0210

GO0210                          STATION DESCRIPTION

GO0210

GO0210'DESCRIBED BY NATIONAL GEODETIC SURVEY 1983

GO0210'14.1 KM (8.75 MI) WEST FROM FARMINGTON.

GO0210'14.1 KM (8.75 MI) WESTERLY ALONG U.S. HIGHWAY 550 FROM ITS JUNCTION

GO0210'WITH STATE HIGHWAY 371 (LAKE STREET) IN FARMINGTON, 0.8 KM (0.5 MI)

GO0210'WEST OF THE INTERSECTION OF STATE HIGHWAY 489 LEADING SOUTH TO THE

GO0210'FOUR CORNERS POWER PLANT, 50.3 METERS (165.0 FT) SOUTHEAST OF THE

GO0210'SOUTHEAST CORNER OF A THRIFTWAY GROCERY AND GAS STATION, 26.1 METERS

GO0210'(85.6 FT) SOUTH OF A UTILITY POLE WITH 1 GUY WIRE, 18.5 METERS

GO0210'(60.7 FT) NORTH OF THE CENTERLINE OF THE WEST BOUND LANES OF THE

GO0210'HIGHWAY, AND 0.6 METERS (2.0 FT) EAST OF THE SOUTH END OF A FENCE.

GO0210'NOTE=ACCESS TO THE DATUM POINT IS THROUGH A 5-INCH LOGO CAP.

GO0210'THE MARK IS 0.3 METERS  W FROM A WITNESS POST.

GO0210'THE MARK IS 0.6 M ABOVE THE HIGHWAY.

GO0210

GO0210                          STATION RECOVERY (1996)

GO0210

GO0210'RECOVERY NOTE BY JE CHANCE AND ASSOCIATES 1996 (MFY)

GO0210'A NEW AND COMPLETE DESCRIPTION FOLLOWS THE STATION IS LOCATED

GO0210'APPROXIMATELY 14.1 KM (8.75 MI) WEST OF THE CITY OF FARMINGTON ALONG

GO0210'US HIGHWAY 64.  THE STATION IS LOCATED ON STATE OF NEW MEXICO HIGHWAY

GO0210'RIGHT-OF-WAY TO REACH THE STATION FROM THE JUNCTION OF US HIGHWAY 550

GO0210'(MAIN STREET) AND US HIGHWAY 64 (BROADWAY STREET) IN FARMINGTON,

GO0210'PROCEED WEST ON US HIGHWAY 64 FOR 12.8 KM (7.95 MI) TO THE

GO0210'INTERSECTION WITH COUNTY ROADS 6500 (NORTH) AND 6400 (SOUTH) AT

GO0210'TRAFFIC LIGHTS. CONTINUE WEST ALONG US HIGHWAY 64 FOR 0.8 KM (0.50 MI)

GO0210'TO THE STATION ON THE RIGHT THE STATION IS A METAL ROD SET INTO A

GO0210'STAINLESS STEEL ROD DRIVEN 3.1 M (10.2 FT) WITHOUT SLEEVE.  THE

GO0210'STATION IS LOCATED 50.3 M (165.0 FT) SOUTHEAST OF THE SOUTHEAST CORNER

GO0210'OF AN ABANDONED GROCERY AND GAS STATION, 26.1 M (85.6 FT) SOUTH OF A

GO0210'UTILITY POLE WITH 1 GUY WIRE, 18.5 M (60.7 FT) NORTH OF THE CENTERLINE

GO0210'OF THE WEST BOUND LANES OF US HIGHWAY 64, 0.6 M (2.0 FT) EAST OF THE

GO0210'SOUTH END OF A FENCE, 0.6 M (2.0 FT) ABOVE US HIGHWAY 64, AND 0.3 M

GO0210'(1.0 FT) WEST OF A WITNESS POST.  NOTE = ACCESS TO DATUM POINT IS

GO0210'THROUGH A 5-INCH LOGO CAP

GO0210

GO0210                          STATION RECOVERY (2006)

GO0210

GO0210'RECOVERY NOTE BY US BUREAU OF RECLAMATION 2006 (TRA)

GO0210'RECOVERED AS DESCRIBED

*** retrieval complete.

Elapsed Time = 00:00:18

## The NGS Data Sheet

##### See file dsdata.txt for more information about the datasheet.

PROGRAM = datasheet95, VERSION = 8.9

1        National Geodetic Survey,   Retrieval Date = SEPTEMBER  7, 2016

DH5816 ***********************************************************************

DH5816  CORS        -  This is a GPS Continuously Operating Reference Station.

DH5816  DESIGNATION -  CHACOCNHP_NM2005 CORS ARP

DH5816  CORS_ID     -  P028

DH5816  PID         -  DH5816

DH5816  STATE/COUNTY-  NM/SAN JUAN

DH5816  COUNTRY     -  US

DH5816  USGS QUAD   -  PUEBLO BONITO (1966)

DH5816

DH5816                         *CURRENT SURVEY CONTROL

DH5816  ______________________________________________________________________

DH5816* NAD 83(2011) POSITION- 36 01 54.04870(N) 107 54 30.22738(W)   ADJUSTED

DH5816* NAD 83(2011) ELLIP HT-  1934.024 (meters)        (08/??/11)   ADJUSTED

DH5816* NAD 83(2011) EPOCH   -  2010.00

DH5816* NAVD 88 ORTHO HEIGHT -          **(meters)           **(feet)

DH5816  ______________________________________________________________________

DH5816  NAD 83(2011) X  - -1,588,369.654 (meters)                     COMP

DH5816  NAD 83(2011) Y  - -4,915,224.209 (meters)                     COMP

DH5816  NAD 83(2011) Z  -  3,732,172.627 (meters)                     COMP

DH5816  GEOID HEIGHT    -        -20.963 (meters)                     GEOID12B

DH5816

DH5816  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

DH5816  Standards:

DH5816         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

DH5816            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

DH5816  -------------------------------------------------------------------

DH5816  NETWORK    0.63   2.04           0.28   0.23   1.04      0.04562330

DH5816  -------------------------------------------------------------------

DH5816  Click here for local accuracies and other accuracy information.

DH5816

DH5816

DH5816.The coordinates were established by GPS observations

DH5816.and adjusted by the National Geodetic Survey in August 2011.

DH5816

DH5816.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

DH5816.been affixed to the stable North American Tectonic Plate.

DH5816

DH5816.The coordinates are valid at the epoch date displayed above

DH5816.which is a decimal equivalence of Year/Month/Day.

DH5816

DH5816.Significant digits in the geoid height do not necessarily reflect accuracy.

DH5816.GEOID12B height accuracy estimate available here.

DH5816

DH5816.The PID for the CORS L1 Phase Center is DH5817.

DH5816

DH5816.The XYZ, and position/ellipsoidal ht. are equivalent.

DH5816

DH5816.The ellipsoidal height was determined by GPS observations

DH5816.and is referenced to NAD 83.

DH5816

DH5816. The following values were computed from the NAD 83(2011) position.

DH5816

DH5816;                    North         East     Units Scale Factor Converg.

DH5816;SPC NM W     -   558,039.659   823,235.300   MT  0.99991723   -0 02 39.0

DH5816;SPC NM W     - 1,830,835.11  2,700,897.81   sFT  0.99991723   -0 02 39.0

DH5816;UTM  13      - 3,991,376.638   237,943.068   MT  1.00044627   -1 42 42.4

DH5816

DH5816!             -  Elev Factor  x  Scale Factor =   Combined Factor

DH5816!SPC NM W     -   0.99969655  x   0.99991723  =   0.99961380

DH5816!UTM  13      -   0.99969655  x   1.00044627  =   1.00014268

DH5816

DH5816                          SUPERSEDED SURVEY CONTROL

DH5816

DH5816  NAD 83(CORS)-  36 01 54.04836(N)    107 54 30.22740(W) AD(2002.00) c

DH5816  ELLIP H (09/??/05) 1934.011  (m)                       GP(2002.00) c c

DH5816

DH5816.Superseded values are not recommended for survey control.

DH5816

DH5816.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

DH5816.See file dsdata.txt to determine how the superseded data were derived.

DH5816

DH5816_U.S. NATIONAL GRID SPATIAL ADDRESS: 13SBV3794391376(NAD 83)

DH5816

DH5816_MARKER: STATION IS THE ANTENNA REFERENCE POINT OF THE GPS ANTENNA

DH5816

DH5816                          STATION DESCRIPTION

DH5816

DH5816'DESCRIBED BY NATIONAL GEODETIC SURVEY 2011

DH5816'STATION IS A GPS CORS.  LATEST INFORMATION INCLUDING POSITIONS AND

DH5816'VELOCITIES ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DH5816'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DH5816'   ftp://cors.ngs.noaa.gov/cors/README.txt

DH5816'   ftp://cors.ngs.noaa.gov/cors/coord/coord_08

DH5816'   ftp://cors.ngs.noaa.gov/cors/station_log

DH5816'   http://geodesy.noaa.gov/CORS

1        National Geodetic Survey,   Retrieval Date = SEPTEMBER  7, 2016